SECTION 09 97 23

ACRYLIC-BASED GLITTER PAINT (CRYSTAL BRUSH® – INTERIOR)

Decorative Glitter Finish – Light-Reflective Sparkle Effect

Notes to Specifier: Information shown in boxes is for specifier use only and should not be included in the final project specifications. Meoded is the premier US manufacturer of decorative coatings, mineral-based paints and lime plasters. Some products are intended for interior use only, while others are formulated for exterior applications, offering resistance to weather, UV exposure, and color stability. With a wide variety of colors and textures, Meoded systems offer versatile design possibilities, featuring a range of sheen levels, visual depth, movement, and natural stone textures. Crystal Brush® is a water-based acrylic glitter coating designed to create a decorative sparkling finish on interior surfaces. The coating incorporates fine glitter particles that require direct or indirect light to produce a visible sparkle effect. The specifier shall select the appropriate product from the Meoded line in accordance with the required color, texture, and finish to achieve the desired result. Modify this specification as needed to suit the project substrates. For assistance with product selection, color samples, or review of project specifications, contact Meoded at (323) 308-2600. Meoded is a manufacturer only and does not perform installation. Responsibility is limited to the manufacture of its decorative finishes and does not extend to defects arising from installation methods, substrate conditions, or other non-manufacturing factors. The following is provided as a general guide for specifying applications of Meoded Crystal Brush® and does not constitute approval of materials or methods not supplied or expressly recommended by Meoded. Installation methods may vary based on field conditions and applicator technique. Application results are dependent upon substrate conditions, environmental factors, and applicator skill, all of which are beyond the manufacturer’s control. Due to the nature of decorative glitter finishes, variation in sparkle intensity, distribution, and visual depth is inherent and expected. Coordinate related sections to ensure proper substrate preparation. Due to the thin application of Crystal Brush®, surface irregularities may be visible. Careful consideration should be given when specifying substrates to clearly define acceptable quality levels. Gypsum Drywall: Provide a Level 4 or Level 5 drywall finish in accordance with the Gypsum Association’s GA-214, Recommended Levels of Gypsum Board Finish. A Level 5 finish is recommended (not required) for areas with critical lighting conditions. Surfaces shall be smooth and uniform; uneven or textured surfaces shall be smoothed as required to achieve consistent glitter distribution. Crystal Brush® may be applied over most smooth plaster substrates, including two- and three-coat gypsum plaster systems, joint compound, and gypsum veneer plaster. Substrates shall be finished to a smooth, steel troweled surface or equivalent. Include expansion and control joints, corner beads, flashings, and all necessary trim components as part of the system. In exterior or moisture-prone areas, specify corrosion-resistant trims and fasteners. Use paper tape exclusively for joint treatment. Crystal Brush® may also be applied over properly prepared, smooth, primed, and painted interior surfaces. Consult Meoded for specific substrate preparation guidelines. Final appearance is dependent on substrate quality, application technique, and lighting conditions required to activate the sparkle effect. Variations in finish are inherent to decorative finishes and should be expected. Substrate preparation and primer selection shall be determined based on substrate type and condition. Substrates shall be clean, dry, stable, and suitable to receive Crystal Brush®.

PART 1 - GENERAL

1.1 SUMMARY

A. Section Includes: Acrylic-based decorative glitter interior coating.

B. Related Sections:

- Section 09 90 00 – Painting and Coating

- Section 09 21 00 - Plaster and Gypsum Board Assemblies

- Additional sections covering substrate materials as applicable to the project

  - REFERENCES

- ASTM E84 – Standard Test Method for Surface Burning Characteristics of Building Materials

- ASTM D3363 – Standard Test Method for Film Hardness by Pencil Test

- ISO 9001 – Quality Management Systems

Referenced standards are provided as general performance benchmarks and do not imply full system compliance unless specifically tested.

1.3 SUBMITTALS

A. Product Data: Submit current manufacturer’s technical data sheets for all specified materials, including installation procedures.

B. Shop Drawings: Provide drawings indicating layout, finish locations, color selections, and interface with adjacent construction.

C. Samples: Submit [two] [_______] sets of representative samples, [minimum 5 x 5 inches]  [_________], demonstrating the full range of selected colors and textures. Samples are intended to illustrate general appearance only. Final installed finish will vary based on application technique, environmental conditions, and individual applicator methods. Samples are not intended to represent exact final appearance.

D. Application Guidelines: Submit manufacturer’s complete application instructions, including surface preparation requirements and installation methods.

E. Maintenance Information: Provide manufacturer’s recommendations for cleaning and maintenance in accordance with Section 01 74 00. Include suggested products, procedures, and maintenance schedules.

1.4 QUALITY ASSURANCE

A. Manufacturer Qualifications: Provide materials manufactured by a company with a minimum of ten (10) years’ experience in the production of decorative plaster and specialty coating systems. Manufacturer shall maintain consistent quality control procedures.

B. Installer Qualifications: Installer shall be experienced in the application of decorative coatings, brush- or spray-applied specialty finishes, and capable of producing work of comparable scope and complexity. Provide documentation of relevant project experience upon request.

C. Technical Support: Manufacturer shall make technical guidance available upon request to assist with product application and installation procedures.

D. Mock-Up:

- Prior to commencement of work, prepare a sample area of each specified finish, approximately [4 feet by 4 feet] [________], at a location designated by the [Architect/Designer]  [_________].

- Mock-up shall demonstrate the intended range of color, texture, and workmanship.

- Approved mock-up shall serve as the standard for the completed work and shall remain in place until project completion. Approved mock-up may be incorporated into the finished work where acceptable. Mock-up establishes the acceptable range of finish characteristics and is not intended to represent an exact uniform finish.

- Final appearance shall be evaluated against the approved mock-up, which establishes acceptable range of texture and movement.

E. Material Consistency: Provide materials supplied by a single manufacturer to ensure consistency in appearance and performance. Use products as recommended by the manufacturer for the specified application.

F. Pre-Installation Conference: Conduct a meeting prior to installation to review substrate conditions, surface preparation requirements, application procedures, and coordination with related trades. Attendees shall include the Contractor, applicator, and other relevant parties. Document and distribute meeting outcomes to all participants.

1.5 DELIVERY, STORAGE, AND HANDLING

A. Delivery and Storage: Deliver materials in original, unopened containers with manufacturer’s labels intact. Protect materials during transport and storage from excessive heat, freezing, and direct sunlight. Store in a cool, dry, and well-ventilated area within a temperature range of 45°F to 95°F.

1.6 WARRANTY

A. Manufacturer’s Limited Warranty: Provide manufacturer’s standard limited warranty for a period of five (5) years from date of installation, covering defects in materials when products are used and applied in accordance with manufacturer’s written instructions.

Warranty is subject to the following conditions:

- Products must be applied within ten (10) months of the date of manufacture.

- Substrates shall be properly constructed and prepared in accordance with industry standards and manufacturer’s recommendations.

- Manufacturer reserves the right to inspect the installation prior to resolution of any warranty claim.

Warranty limitations:
This warranty is limited to replacement of defective material or refund of the original purchase price, at the manufacturer’s discretion, upon verification of a valid claim.

This warranty does not cover:

- Improper substrate preparation

- Installation or workmanship errors

- Cracking, movement, or structural conditions

- Aesthetic variations or natural color changes including variations resulting from hand application techniques and environmental conditions.

- Water intrusion or moisture-related issues

- Labor or costs associated with removal or reinstallation

- Damage caused by misuse, neglect, or acts of nature

This warranty is exclusive and in lieu of all other warranties, whether express or implied. Manufacturer shall not be liable for incidental or consequential damages.

1.7 ENVIRONMENTAL CONDITIONS

• Apply only when ambient and surface temperatures are between 50°F and 90°F
• Relative humidity between 25% and 75%
• Provide adequate ventilation during application

  - SYSTEM LIMITATIONS

• Crystal Brush® is a decorative coating intended for aesthetic purposes only.

• Application is limited to brush or spray methods; roller application alone is not permitted.

• If roller is used for material placement, it shall be immediately followed by back-brushing.

• Glitter effect is dependent on lighting; insufficient lighting will reduce or eliminate visible sparkle.

• Due to the nature of glitter finishes, uniform sparkle distribution cannot be guaranteed.

• Brush application will produce visible brush marks; spray application will produce a more uniform appearance.

• Substrate imperfections may be visible depending on lighting conditions.

• Finish is not washable.

• Protective sealers are not required and are not recommended, as they may dull or obscure the glitter effect.

• Final appearance is dependent on substrate quality, application technique, environmental conditions, and lighting.

• Variations in appearance may occur at transitions, edges, and changes in substrate conditions.

• Appearance will vary significantly depending on light source, intensity, and viewing angle.

• Finish is decorative in nature and not intended for high-traffic or high-abrasion areas.

• Lighting design should be considered during specification to achieve desired sparkle effect.

PART 2 - PRODUCTS

2.1 APPROVED MANUFACTURER:

Meoded Paints & Decoration, Inc.

6314 Santa Monica Blvd, Los Angeles, CA 90038

P: 323-308-2600

E: info@meodedpaint.com

www.meodedPaint.com

2.2 SYSTEM PERFORMANCE REQUIREMENTS

A. Material Composition: Water-based acrylic coating containing decorative glitter particles.

B. VOC Content: Low VOC coating (≤ 50 g/L; actual ~48 g/L)

C. Washability: Coating is not washable.

D. Appearance Characteristics: Light-reflective glitter finish requiring light exposure to produce sparkle effect.

2.3 FINISHES

A. Product: Meoded Crystal Brush® glitter paint system.

B. Finish: Decorative glitter coating.

C. Selection:

- Colors shall be selected from the manufacturer's standard palette of glitter paint colors.

- Custom colors available upon request.

- Custom color matching may result in slight variation from submitted samples.

D. Aesthetic Variation: Finish shall exhibit variation in sparkle intensity, distribution, and reflectivity inherent to glitter coatings.

2.4 ACCESSORIES

A. Primer: Provide substrate-specific primers (PVA, multipurpose, metal, wood) as required.

B. Protective Finish: Not required. Not recommended.

PART 3 - EXECUTION

3.1 EXAMINATION

A. Verify substrates are suitable for application.
B. Do not proceed until unsatisfactory conditions have been corrected.

3.2 PREPARATION

A. Protect adjacent surfaces not scheduled to receive finish.

B. Substrate Preparation:

- Existing Substrates:

a. Deteriorated surfaces shall be repaired or replaced to provide a sound substrate.

b. Oily, glossy, or previously painted surfaces shall be lightly sanded to promote adhesion.

c. Clean surfaces using appropriate methods to remove dirt, dust, grease, contaminants, and any substances that may interfere with adhesion or appearance.

d. Prime using a manufacturer-approved primer compatible with the substrate type (e.g., wood primer for wood, metal primer for metal, multipurpose primer for previously painted surfaces).

e. Surfaces shall be smooth and uniform. Uneven or textured surfaces shall be floated or skim-coated as required to achieve a Level 4 or Level 5–equivalent finish suitable for a brush- or spray-applied decorative coating.

2.   New Substrates:

a. Gypsum wallboard shall have a Level 4 or Level 5 finish and be primed with a PVA primer to achieve a uniform surface.

3.  General Requirements:

a. All substrates shall be clean, dry, and free of dust, dirt, oil, grease, efflorescence, and other contaminants that may affect adhesion or appearance.

b. Substrate shall be suitable to achieve the aesthetic intent of the finish as approved in mock-up and consistent with manufacturer’s recommendations. Uneven or textured surfaces shall be floated smooth prior to application to achieve a uniform decorative finish.

c. Priming: Apply the proper primer depending on the substrate as recommended by the manufacturer.

3.3 MIXING

A. Stir material thoroughly prior to application.
B. No water addition required.

3.4 APPLICATION TOOLS

A. Use high-quality brushes or spray equipment (HVLP recommended).

B. Recommended spray tip size: 3–4 (HVLP).

3.5 APPLICATION

A. Apply materials in accordance with manufacturer’s written instructions.

B. Apply in two thin coats to achieve specified texture and appearance.

C. Process:

- Apply using brush in a cross-hatch pattern to achieve organic sparkle variation.

- Roller may be used only for initial placement and must be immediately back-brushed.

- Spray application produces the most uniform sparkle appearance.

D. Technique: Application technique directly affects sparkle distribution and final appearance.

E. Brush application will result in visible directional marks; spray will minimize patterning.

F. Maintain a wet edge during application to minimize lap marks and ensure uniform appearance.

G. Maintain consistent application technique and sequencing across continuous surfaces.

3.6 SEALER APPLICATION

A. No sealer or protective topcoat shall be applied.

3.7 CLEANING

A. Remove masking and protection materials.

B. Clean adjacent surfaces without damage.

C. Clean tools immediately after use.

3.8 PROTECTION

A. Protect finished surfaces from damage during construction. Avoid contact until fully cured.

B. Protect finished surfaces from excessive contact or abrasion prior to full cure.

3.9 AESTHETIC INTENT

A. Finish shall exhibit a decorative glitter sparkle appearance activated by light.
B. Surface shall display variation in sparkle intensity depending on lighting conditions and viewing angle.
C. Variation in glitter distribution is inherent and not a defect.

© Meoded Paints & Decoration, Inc. Document may be copied by design and construction professionals for use in preparing project specifications for Meoded products. Meoded and product names are trademarks of Meoded Paints & Decoration, INC.

Revision Date: April 2026