SECTION 09 97 23

ACRYLIC-BASED METALLIC PAINT (SAPPHIRE METALLIC® – EXTERIOR)

Decorative Metallic Finish – Natural Mica-Based Shimmer Effect

Notes to Specifier: Information shown in boxes is for specifier use only and should not be included in the final project specifications. Meoded is the premier US manufacturer of decorative coatings, mineral-based paints and lime plasters. Some products are intended for interior use only, while others are formulated for exterior applications, offering resistance to weather, UV exposure, and color stability. With a wide variety of colors and textures, Meoded systems offer versatile design possibilities, featuring a range of sheen levels, visual depth, movement, and natural stone textures. The specifier shall select the appropriate product from the Meoded line in accordance with the required color, texture, sheen, and finish to achieve the desired result. Modify this specification as needed to suit the project substrates. For assistance with product selection, color samples, or review of project specifications, contact Meoded at (323) 308-2600. Meoded is a manufacturer only and does not perform installation. Responsibility is limited to the manufacture of its decorative finishes and does not extend to defects arising from installation methods, substrate conditions, or other non-manufacturing factors. The following is provided as a general guide for specifying applications of Meoded Sapphire Metallic® and does not constitute approval of materials or methods not supplied or expressly recommended by Meoded. Installation methods may vary based on field conditions and applicator technique. Application results are dependent upon substrate conditions, environmental factors, and applicator skill, all of which are beyond the manufacturer’s control. Sapphire Metallic® may be applied over properly prepared exterior substrates including: • Cement plaster (stucco) • Concrete and masonry • Wood • Metal • Previously painted exterior surfaces Substrate preparation and primer selection shall be determined based on substrate type and condition. Due to the reflective nature of metallic finishes, surface irregularities, joint lines, and application marks may be visible. Exterior applications require proper substrate design, water management detailing, and protection from environmental exposure during installation and curing. Coordinate related sections to ensure proper substrate preparation. Careful consideration should be given when specifying substrates to clearly define acceptable quality levels. Substrates shall be clean, dry, stable, and suitable to receive Sapphire Metallic®. Final appearance is dependent on substrate quality, application technique, environmental exposure, and lighting conditions.

PART 1 - GENERAL

1.1 SUMMARY

A. Section Includes: Acrylic-based metallic decorative exterior paint finish

B. Related Sections:

- Section 09 90 00 – Painting and Coating

- Section 07 00 00 – Thermal and Moisture Protection

- Additional sections covering substrate materials as applicable to the project

  - REFERENCES

- ASTM E84 – Standard Test Method for Surface Burning Characteristics of Building Materials

- ASTM D2486 – Standard Test Method for Scrub Resistance of Wall Coatings

- ASTM D3363 – Standard Test Method for Film Hardness by Pencil Test

- ISO 9001 – Quality Management Systems

Referenced standards are provided as general performance benchmarks and do not imply full system compliance unless specifically tested.

1.3 SUBMITTALS

A. Product Data: Submit current manufacturer’s technical data sheets for all specified materials, including installation procedures.

B. Shop Drawings: Provide drawings indicating layout, finish locations, color selections, and interface with adjacent construction.

C. Samples: Submit [two] [_______] sets of representative samples, [minimum 5 x 5 inches]  [_________], demonstrating the full range of selected colors and textures. Samples are intended to illustrate general appearance only. Final installed finish will vary based on application technique, environmental conditions, and individual applicator methods. Samples are not intended to represent exact final appearance.

D. Application Guidelines: Submit manufacturer’s complete application instructions, including surface preparation requirements and installation methods.

E. Maintenance Information: Provide manufacturer’s recommendations for cleaning and maintenance in accordance with Section 01 74 00. Include suggested products, procedures, and maintenance schedules.

1.4 QUALITY ASSURANCE

A. Manufacturer Qualifications: Provide materials manufactured by a company with a minimum of ten (10) years’ experience in the production of decorative plaster and specialty coating systems. Manufacturer shall maintain consistent quality control procedures.

B. Installer Qualifications: Installer shall be experienced in the application of decorative coatings or similar specialty finishes and capable of producing work of comparable scope and complexity. Provide documentation of relevant project experience upon request.

C. Technical Support: Manufacturer shall make technical guidance available upon request to assist with product application and installation procedures.

D. Mock-Up:

- Prior to commencement of work, prepare a sample area of each specified finish, approximately [4 feet by 4 feet] [________], at a location designated by the [Architect/Designer]  [_________].

- Mock-up shall demonstrate the intended range of color, texture, and workmanship.

- Approved mock-up shall serve as the standard for the completed work and shall remain in place until project completion. Approved mock-up may be incorporated into the finished work where acceptable. Mock-up establishes the acceptable range of finish characteristics and is not intended to represent an exact uniform finish.

E. Material Consistency: Provide materials supplied by a single manufacturer to ensure consistency in appearance and performance. Use products as recommended by the manufacturer for the specified application.

F. Pre-Installation Conference: Conduct a meeting prior to installation to review substrate conditions, surface preparation requirements, application procedures, and coordination with related trades. Attendees shall include the Contractor, applicator, and other relevant parties. Document and distribute meeting outcomes to all participants.

1.5 DELIVERY, STORAGE, AND HANDLING

A. Delivery and Storage: Deliver materials in original, unopened containers with manufacturer’s labels intact. Protect materials during transport and storage from excessive heat, freezing, and direct sunlight. Store in a cool, dry, and well-ventilated area within a temperature range of 45°F to 95°F.

1.6 WARRANTY

A. Manufacturer’s Limited Warranty: Provide manufacturer’s standard limited warranty for a period of five (5) years from date of installation, covering defects in materials when products are used and applied in accordance with manufacturer’s written instructions.

Warranty is subject to the following conditions:

- Products must be applied within ten (10) months of the date of manufacture.

- Substrates shall be properly constructed and prepared in accordance with industry standards and manufacturer’s recommendations.

- Manufacturer reserves the right to inspect the installation prior to resolution of any warranty claim.

Warranty limitations:
This warranty is limited to replacement of defective material or refund of the original purchase price, at the manufacturer’s discretion, upon verification of a valid claim.

This warranty does not cover:

- Improper substrate preparation

- Installation or workmanship errors

- Cracking, movement, or structural conditions

- Aesthetic variations or natural color changes including variations resulting from hand application techniques and environmental conditions.

- Water intrusion or moisture-related issues

- Labor or costs associated with removal or reinstallation

- Damage caused by misuse, neglect, or acts of nature

This warranty is exclusive and in lieu of all other warranties, whether express or implied. Manufacturer shall not be liable for incidental or consequential damages.

1.7 ENVIRONMENTAL CONDITIONS

• Apply only when ambient and surface temperatures are between 45°F and 95°F
• Do not apply during rain, fog, or high humidity conditions 
• Do not apply when precipitation is expected within 24 hours 
• Protect from wind, direct sun, and rapid drying during application

• Do not apply to surfaces exceeding manufacturer’s recommended temperature limits.

  - SYSTEM LIMITATIONS

• Metallic finishes are decorative coatings intended for aesthetic purposes. 
• Reflective pigments may highlight surface imperfections. 
• Brush and roller application may result in visible directional patterns. 
• Spray application is recommended for most uniform appearance.

• Exterior exposure will result in natural weathering over time.

• Finish is not intended as a waterproofing system.
• Finish is washable, but not intended for heavy abrasion. 
• Variation in metallic movement and reflectivity is inherent.

• Final appearance may vary based on lighting conditions, substrate uniformity, application method, and environmental exposure.

• Variations in sheen and reflectivity may occur at transitions, edges, and changes in substrate conditions.

• Uniform appearance is dependent on consistent application technique and sequencing.

• Metallic finishes may exhibit variation in appearance under different lighting conditions including natural and artificial light sources.

PART 2 - PRODUCTS

2.1 APPROVED MANUFACTURER:

Meoded Paints & Decoration, Inc.

6314 Santa Monica Blvd, Los Angeles, CA 90038

P: 323-308-2600

E: info@meodedpaint.com

www.meodedPaint.com

2.2 SYSTEM PERFORMANCE REQUIREMENTS

A. Material Composition: Acrylic-based decorative coating composed of water-based resins and natural mica powders.

B. VOC Content: Low VOC coating (≤ 50 g/L; actual ~20.13 g/L)

C. Weather Resistance: Coating shall be suitable for exterior exposure when applied and sealed in accordance with manufacturer’s recommendations.

D. Appearance Characteristics: Metallic shimmer and reflective movement depending on lighting.

2.3 FINISHES

A. Product: Meoded Sapphire Metallic® metallic paint system

B. Finish: Metallic, reflective decorative coating

C. Selection:

- Colors shall be selected from the manufacturer's standard palette of approximately 60 metallic colors.

- Custom colors available upon request.

- Custom color matching may result in slight variation from submitted samples.

D. Aesthetic Variation: Variation in reflectivity, movement, and tone is inherent to metallic finishes and shall not be considered a defect.

2.4 ACCESSORIES

A. Primer: Provide substrate-specific primers (PVA, multipurpose, metal, wood) as required.

B. Protective Finish (Recommended): Meoded Cristallo Clear protective topcoat.

- Recommended where additional washability or protection is required.

- Matte or Semi-Gloss options.

- Semi-Gloss enhances metallic effect.

- Matte softens sheen.

PART 3 - EXECUTION

3.1 EXAMINATION

A. Verify substrates are suitable for application.
B. Do not proceed until unsatisfactory conditions have been corrected.

3.2 PREPARATION

A. Protect adjacent surfaces not scheduled to receive finish.

B. Substrate Preparation:

- Exterior Cementitious Substrates:

a. Surfaces shall be fully cured, clean, and free of contaminants.

b. Repair cracks and surface defects prior to application

c. Clean surfaces using appropriate methods to remove dirt, dust, grease, contaminants, and any substances that may interfere with adhesion or appearance.

d. Prime using a manufacturer-recommended primer compatible with the substrate type.

2.   Previously Painted Exterior Surfaces:

- Remove loose or peeling coatings

- Sand glossy surfaces

c.   Use a multipurpose primer compatible with previously painted surfaces.

3.   Wood and Metal Substrates:

a.   Apply manufacturer-recommended exterior-grade primer appropriate for the specific substrate (wood or metal).

4.   General Requirements:

a. All substrates shall be clean, dry, and free of dust, dirt, oil, grease, efflorescence, and other contaminants that may affect adhesion or appearance.

b. Substrate shall be suitable to achieve the aesthetic intent of the finish as approved in mock-up and consistent with manufacturer’s recommendations.

c. Do not apply over substrates with active moisture intrusion

d. Verify proper flashing and drainage systems

3.3 MIXING

A. Stir material thoroughly prior to application.
B. No water addition required.

3.4 APPLICATION

A. Apply using brush, roller, or spray equipment.

B. Spray application recommended for uniform metallic appearance.

C. Apply in 2–3 coats for full coverage and exterior performance

D. Application technique directly affects final appearance.

E. Multiple colors may be layered/blended for effects.

F. Maintain a wet edge during application to minimize lap marks and ensure uniform appearance.

G. Apply material in a consistent direction and sequence to maintain uniform metallic appearance across the surface.

H. Protect from environmental exposure during application and curing.

I. Do not apply under direct sunlight or on hot substrates that may cause rapid drying.

J. Maintain continuous application across wall sections to avoid visible start/stop lines.

3.5 PROTECTIVE TOPCOAT

A. Cristallo Clear topcoat is not required for standard exterior applications but may be specified where additional washability, durability, or protection is desired.

B. When specified, apply 2–3 coats in accordance with manufacturer’s instructions.

C. Topcoat provides additional resistance to moisture, abrasion, and environmental exposure.

D. Topcoat may alter sheen level and visual depth depending on selected finish.

3.6 CLEANING

A. Remove masking and protection materials.

B. Clean adjacent surfaces without damage.

C. Clean tools immediately after use.

3.7 PROTECTION

A. Protect finished surfaces from damage during construction. Avoid contact until fully cured.

B. Protect from rain, moisture, and weather exposure during curing.

3.8 AESTHETIC INTENT

A. Finish shall exhibit metallic shimmer.
B. Appearance varies based on lighting.
C. Non-uniformity is inherent.
D. Directional patterns or variation are not defects.

E. Weathering and environmental exposure may alter appearance over time and shall not be considered a defect

© Meoded Paints & Decoration, Inc. Document may be copied by design and construction professionals for use in preparing project specifications for Meoded products. Meoded and product names are trademarks of Meoded Paints & Decoration, INC.

Revision Date: April 2026