SECTION 09 97 23

ACRYLIC-BASED SUEDE METALLIC PAINT (PEARLAS VELVET® – INTERIOR)

Decorative Suede Metallic Finish – Velvety Venetian Plaster Appearance

Notes to Specifier: Information shown in boxes is for specifier use only and should not be included in the final project specifications. Meoded is the premier US manufacturer of decorative coatings, mineral-based paints and lime plasters. Some products are intended for interior use only, while others are formulated for exterior applications, offering resistance to weather, UV exposure, and color stability. With a wide variety of colors and textures, Meoded systems offer versatile design possibilities, featuring a range of sheen levels, visual depth, movement, and natural stone textures. Pearlas Velvet® is a water-based acrylic suede metallic coating designed to replicate the appearance of a soft suede Venetian plaster finish. The specifier shall select the appropriate product from the Meoded line in accordance with the required color, texture, sheen, and finish to achieve the desired result. Modify this specification as needed to suit the project substrates. For assistance with product selection, color samples, or review of project specifications, contact Meoded at (323) 308-2600. Meoded is a manufacturer only and does not perform installation. Responsibility is limited to the manufacture of its decorative finishes and does not extend to defects arising from installation methods, substrate conditions, or other non-manufacturing factors. The following is provided as a general guide for specifying applications of Meoded Pearlas Velvet® and does not constitute approval of materials or methods not supplied or expressly recommended by Meoded. Installation methods may vary based on field conditions and applicator technique. Due to the trowel-applied suede nature of the finish, variations in movement, texture, and tonal depth are inherent and expected. Application results are dependent upon substrate conditions, environmental factors, and applicator skill, all of which are beyond the manufacturer’s control. Coordinate related sections to ensure proper substrate preparation. Due to the thin application of Pearlas Velvet®, surface irregularities may be visible. Careful consideration should be given when specifying substrates to clearly define acceptable quality levels. Gypsum Drywall: Provide a Level 5 drywall finish in accordance with the Gypsum Association’s GA-214, Recommended Levels of Gypsum Board Finish. A Level 5 finish offers the highest quality surface and is recommended for areas with critical lighting conditions, large wall expanses, and where smooth or gloss finishes are specified. Pearlas Velvet® may be applied over most smooth plaster substrates, including two- and three-coat gypsum plaster systems, joint compound, and gypsum veneer plaster. Substrates shall be finished to a smooth, steel troweled surface or equivalent. Include expansion and control joints, corner beads, flashings, and all necessary trim components as part of the system. In exterior or moisture-prone areas, specify corrosion-resistant trims and fasteners. Use paper tape exclusively for joint treatment. Pearlas Velvet® may also be applied over properly prepared, smooth, primed, and painted interior surfaces. Consult Meoded for specific substrate preparation guidelines. Final appearance is dependent on substrate quality, application technique, and field conditions. Variations in finish are inherent to hand-applied decorative finishes and should be expected. Substrate preparation and primer selection shall be determined based on substrate type and condition. Substrates shall be clean, dry, stable, and suitable to receive Pearlas Velvet®.

PART 1 - GENERAL

1.1 SUMMARY

A. Section Includes: Acrylic-based suede metallic decorative interior finish

B. Related Sections:

- Section 09 90 00 – Painting and Coating

- Section 09 21 00 - Plaster and Gypsum Board Assemblies

- Additional sections covering substrate materials as applicable to the project

  - REFERENCES

- ASTM E84 – Standard Test Method for Surface Burning Characteristics of Building Materials

- ASTM D2486 – Standard Test Method for Scrub Resistance of Wall Coatings

- ASTM D3363 – Standard Test Method for Film Hardness by Pencil Test

- ISO 9001 – Quality Management Systems

Referenced standards are provided as general performance benchmarks and do not imply full system compliance unless specifically tested.

1.3 SUBMITTALS

A. Product Data: Submit current manufacturer’s technical data sheets for all specified materials, including installation procedures.

B. Shop Drawings: Provide drawings indicating layout, finish locations, color selections, and interface with adjacent construction.

C. Samples: Submit [two] [_______] sets of representative samples, [minimum 5 x 5 inches]  [_________], demonstrating the full range of selected colors and textures. Samples are intended to illustrate general appearance only. Final installed finish will vary based on application technique, environmental conditions, and individual applicator methods. Samples are not intended to represent exact final appearance.

D. Application Guidelines: Submit manufacturer’s complete application instructions, including surface preparation requirements and installation methods.

E. Maintenance Information: Provide manufacturer’s recommendations for cleaning and maintenance in accordance with Section 01 74 00. Include suggested products, procedures, and maintenance schedules.

1.4 QUALITY ASSURANCE

A. Manufacturer Qualifications: Provide materials manufactured by a company with a minimum of ten (10) years’ experience in the production of decorative plaster and specialty coating systems. Manufacturer shall maintain consistent quality control procedures.

B. Installer Qualifications: Installer shall be experienced in the application of decorative coatings, including trowel-applied specialty finishes, and capable of producing work of comparable scope and complexity. Provide documentation of relevant project experience upon request.

C. Technical Support: Manufacturer shall make technical guidance available upon request to assist with product application and installation procedures.

D. Mock-Up:

- Prior to commencement of work, prepare a sample area of each specified finish, approximately [4 feet by 4 feet] [________], at a location designated by the [Architect/Designer]  [_________].

- Mock-up shall demonstrate the intended range of color, texture, and workmanship.

- Approved mock-up shall serve as the standard for the completed work and shall remain in place until project completion. Approved mock-up may be incorporated into the finished work where acceptable. Mock-up establishes the acceptable range of finish characteristics and is not intended to represent an exact uniform finish.

- Final appearance shall be evaluated against the approved mock-up, which establishes acceptable range of texture and movement.

E. Material Consistency: Provide materials supplied by a single manufacturer to ensure consistency in appearance and performance. Use products as recommended by the manufacturer for the specified application.

F. Pre-Installation Conference: Conduct a meeting prior to installation to review substrate conditions, surface preparation requirements, application procedures, and coordination with related trades. Attendees shall include the Contractor, applicator, and other relevant parties. Document and distribute meeting outcomes to all participants.

1.5 DELIVERY, STORAGE, AND HANDLING

A. Delivery and Storage: Deliver materials in original, unopened containers with manufacturer’s labels intact. Protect materials during transport and storage from excessive heat, freezing, and direct sunlight. Store in a cool, dry, and well-ventilated area within a temperature range of 45°F to 95°F.

1.6 WARRANTY

A. Manufacturer’s Limited Warranty: Provide manufacturer’s standard limited warranty for a period of five (5) years from date of installation, covering defects in materials when products are used and applied in accordance with manufacturer’s written instructions.

Warranty is subject to the following conditions:

- Products must be applied within ten (10) months of the date of manufacture.

- Substrates shall be properly constructed and prepared in accordance with industry standards and manufacturer’s recommendations.

- Manufacturer reserves the right to inspect the installation prior to resolution of any warranty claim.

Warranty limitations:
This warranty is limited to replacement of defective material or refund of the original purchase price, at the manufacturer’s discretion, upon verification of a valid claim.

This warranty does not cover:

- Improper substrate preparation

- Installation or workmanship errors

- Cracking, movement, or structural conditions

- Aesthetic variations or natural color changes including variations resulting from hand application techniques and environmental conditions.

- Water intrusion or moisture-related issues

- Labor or costs associated with removal or reinstallation

- Damage caused by misuse, neglect, or acts of nature

This warranty is exclusive and in lieu of all other warranties, whether express or implied. Manufacturer shall not be liable for incidental or consequential damages.

1.7 ENVIRONMENTAL CONDITIONS

• Apply only when ambient and surface temperatures are between 45°F and 95°F
• Do not apply under high humidity or poor drying conditions
• Provide adequate ventilation during application

  - SYSTEM LIMITATIONS

• Pearlas Velvet® is a decorative coating intended for aesthetic purposes only.

• Application is limited to trowel or spatula methods only; brush, roller, or spray application is not permitted.

• Due to the thin, hand-troweled application, surface irregularities, joints, and substrate imperfections may be visible and telegraphed through the finish.

• A Level 5 substrate finish is required to achieve the intended aesthetic result, particularly in areas with critical lighting.

• Reflective and tonal variation is inherent to metallic finishes and shall not be considered a defect.

• Finish exhibits metallic movement; appearance will vary depending on lighting conditions and viewing angle.

• Trowel application produces natural variation in texture, movement, and tonal depth; uniform appearance is not achievable or intended.

• Variations in trowel pattern, pressure, and technique will affect final appearance and are inherent to the system.

• Finish is washable under normal interior conditions.

• Protective sealers are not required and are not recommended, as they may alter the intended suede appearance.

• Final appearance is dependent on substrate quality, application technique, environmental conditions, and lighting.

• Variations in appearance may occur at transitions, edges, and changes in substrate conditions.

• Appearance may vary significantly under different lighting conditions, including natural and artificial light sources.

• Minor trowel marks and variations in finish are inherent and shall not be considered defects.

PART 2 - PRODUCTS

2.1 APPROVED MANUFACTURER:

Meoded Paints & Decoration, Inc.

6314 Santa Monica Blvd, Los Angeles, CA 90038

P: 323-308-2600

E: info@meodedpaint.com

www.meodedPaint.com

2.2 SYSTEM PERFORMANCE REQUIREMENTS

A. Material Composition: Water-based acrylic coating composed of metallic pigments and fine resins designed to produce a suede metallic finish.

B. VOC Content: Low VOC coating (≤ 50 g/L; actual ~21.596 g/L)

C. Washability: Coating is washable under normal interior conditions.

D. Appearance Characteristics: Soft suede metallic appearance with subtle reflective qualities and multi-tone depth.

2.3 FINISHES

A. Product: Meoded Pearlas Velvet® suede metallic paint system

B. Finish: Suede metallic Venetian plaster appearance.

C. Selection:

- Colors shall be selected from the manufacturer's standard palette of suede metallic colors.

- Custom colors available upon request.

- Custom color matching may result in slight variation from submitted samples.

D. Aesthetic Variation: Finish shall exhibit variation in tone, texture, reflectivity, and metallic movement inherent to hand trowel metallic finishes and shall not be considered a defect.

2.4 ACCESSORIES

A. Primer: Provide substrate-specific primers (PVA, multipurpose, metal, wood) as required.

B. Protective Finish: Not required. Not recommended (finish is inherently washable).

PART 3 - EXECUTION

3.1 EXAMINATION

A. Verify substrates are suitable for application.
B. Do not proceed until unsatisfactory conditions have been corrected.

3.2 PREPARATION

A. Protect adjacent surfaces not scheduled to receive finish.

B. Substrate Preparation:

- Existing Substrates:

a. Deteriorated surfaces shall be repaired or replaced to provide a sound substrate.

b. Oily, glossy, or previously painted surfaces shall be lightly sanded to promote adhesion.

c. Clean surfaces using appropriate methods to remove dirt, dust, grease, contaminants, and any substances that may interfere with adhesion or appearance.

d. Prime using a manufacturer-approved primer compatible with the substrate type (e.g., wood primer for wood, metal primer for metal, multipurpose primer for previously painted surfaces).

e. Surfaces shall be smooth and uniform. Uneven or textured surfaces shall be floated or skim-coated as required to achieve a Level 5–equivalent finish suitable for a trowel-applied decorative coating.

2.   New Substrates:

a. Gypsum wallboard shall have a Level 5 finish and be primed with a PVA primer to achieve a uniform surface.

3.  General Requirements:

a. All substrates shall be clean, dry, and free of dust, dirt, oil, grease, efflorescence, and other contaminants that may affect adhesion or appearance.

b. Substrate shall be suitable to achieve the aesthetic intent of the finish as approved in mock-up and consistent with manufacturer’s recommendations. Uneven or textured surfaces shall be floated smooth prior to application to achieve refined suede finish.

c. Priming: Apply the proper primer depending on the substrate as recommended by the manufacturer.

3.3 MIXING

A. Stir material thoroughly prior to application.
B. No water addition required.

3.4 APPLICATION TOOLS

A. Use stainless steel trowels or spatulas only

B. Tools shall match approved mock-up

3.5 APPLICATION

A. Apply materials in accordance with manufacturer’s written instructions.

B. Apply in two thin coats to achieve specified texture and appearance.

C. Process:

- Apply first coat using varied trowel strokes

- Allow minimum of one (1) hour drying time between coats, or as required by environmental conditions.

- Apply second coat with varied movement

- Use clean trowel to refine finish and achieve desired movement

D. Technique: Apply material in small amounts using varied directional strokes to create a suede Venetian plaster appearance.

E. Multiple colors may be layered/blended for effects.

F. Maintain a wet edge during application to minimize lap marks and ensure uniform appearance.

G. Use clean tools during finishing passes to avoid unwanted marks or discoloration.

3.6 SEALER APPLICATION

A. No sealer or protective topcoat shall be applied.

3.7 CLEANING

A. Remove masking and protection materials.

B. Clean adjacent surfaces without damage.

C. Clean tools immediately after use.

3.8 PROTECTION

A. Protect finished surfaces from damage during construction. Avoid contact until fully cured.

B. Protect finished surfaces from excessive contact or abrasion prior to full cure.

3.9 AESTHETIC INTENT

A. Finish shall exhibit a soft suede metallic appearance.
B. Surface shall display trowel movement and depth.
C. Finish shall display multi-tone color variation under different lighting conditions.
D. Variation is inherent and not a defect

© Meoded Paints & Decoration, Inc. Document may be copied by design and construction professionals for use in preparing project specifications for Meoded products. Meoded and product names are trademarks of Meoded Paints & Decoration, INC.

Revision Date: April 2026