SECTION 09 97 23

LIME-BASED MINERAL PAINT (LIME WASH - INTERIOR)

Natural Matte Mineral Finish – Subtle Movement and Patina Effect

Notes to Specifier: Information shown in boxes is for specifier use only and should not be included in the final project specifications. Meoded is the premier US manufacturer of natural mineral based paints and lime plasters. Some products are intended for interior use only, while others are formulated for exterior applications, offering resistance to weather, UV exposure, and color stability. With a wide variety of colors and textures, Meoded systems offer versatile design possibilities, featuring a range of sheen levels, visual depth, movement, and natural stone textures. The specifier shall select the appropriate product from the Meoded line in accordance with the required color, texture, sheen, and finish to achieve the desired result. Modify this specification as needed to suit the project substrates. For assistance with product selection, color samples, or review of project specifications, contact Meoded at (323) 308-2600. Meoded is a manufacturer only and does not perform installation. Responsibility is limited to the manufacture of its decorative finishes and does not extend to defects arising from installation methods, substrate conditions, or other non-manufacturing factors. The following is provided as a general guide for specifying applications of Meoded Lime Wash and does not constitute approval of materials or methods not supplied or expressly recommended by Meoded. Installation methods may vary based on field conditions and applicator technique. Application results are dependent upon substrate conditions, environmental factors, and applicator skill, all of which are beyond the manufacturer’s control. Coordinate related sections to ensure proper substrate preparation. Due to the translucent and matte nature of lime wash finishes, surface irregularities and substrate variations may be visible. Careful consideration should be given when specifying substrates to clearly define acceptable quality levels. Interior Substrates: Lime Wash may be applied over properly prepared interior substrates including: - Gypsum drywall (Level 4 or Level 5 finish) - Interior plaster - Concrete and masonry - Previously painted surfaces (properly prepared and primed) Substrates shall be clean, dry, stable, and suitable to receive mineral-based coatings. Lime Wash requires application of a compatible mineral base coat primer to regulate absorption and ensure proper adhesion. Final appearance is dependent on substrate quality, application technique, and field conditions. Variations in finish, tonal movement, and patina development are inherent to lime wash systems and should be expected.

PART 1 - GENERAL

1. SUMMARY

A. Section Includes: Lime-based mineral paint interior finish system.

B. Related Sections:

- Section 09 21 00 - Plaster and Gypsum Board Assemblies

- Section 09 90 00 – Painting and Coating

- Additional sections covering substrate materials as applicable to the project

  - REFERENCES

A. ASTM E84 – Standard Test Method for Surface Burning Characteristics of Building Materials
B. ISO 9001 – Quality Management Systems

Referenced standards are provided as general performance benchmarks and do not imply full system compliance unless specifically tested.

1.3 SUBMITTALS

A. Product Data: Submit current manufacturer’s technical data sheets for all specified materials, including installation procedures.

B. Shop Drawings: Provide drawings indicating layout, finish locations, color selections, and interface with adjacent construction.

C. Samples: Submit [two] [_______] sets of representative samples, [minimum 5 x 5 inches]  [_________], demonstrating the full range of selected colors and textures. Samples are intended to illustrate general appearance only. Final installed finish will vary based on application technique, environmental conditions, and individual applicator methods. Samples are not intended to represent exact final appearance.

D. Application Guidelines: Submit manufacturer’s complete application instructions, including surface preparation requirements and installation methods.

G. Maintenance Information: Provide manufacturer’s recommendations for cleaning and maintenance in accordance with Section 01 74 00. Include suggested products, procedures, and maintenance schedules.

1.4 QUALITY ASSURANCE

A. Manufacturer Qualifications: Provide materials manufactured by a company with a minimum of ten (10) years’ experience in the production of decorative plaster and specialty coating systems. Manufacturer shall maintain consistent quality control procedures.

B. Installer Qualifications: Installer shall be experienced in the application of decorative coatings or similar specialty finishes and capable of producing work of comparable scope and complexity. Provide documentation of relevant project experience upon request.

C. Technical Support: Manufacturer shall make technical guidance available upon request to assist with product application and installation procedures.

D. Mock-Up:

- Prior to commencement of work, prepare a sample area of each specified finish, approximately [4 feet by 4 feet] [________], at a location designated by the [Architect/Designer]  [_________].

- Mock-up shall demonstrate the intended range of color, texture, and workmanship.

- Approved mock-up shall serve as the standard for the completed work and shall remain in place until project completion. Approved mock-up may be incorporated into the finished work where acceptable. Mock-up establishes the acceptable range of finish characteristics and is not intended to represent an exact uniform finish.

E. Material Consistency: Provide materials supplied by a single manufacturer to ensure consistency in appearance and performance. Use products as recommended by the manufacturer for the specified application.

F. Pre-Installation Conference: Conduct a meeting prior to installation to review substrate conditions, surface preparation requirements, application procedures, and coordination with related trades. Attendees shall include the Contractor, applicator, and other relevant parties. Document and distribute meeting outcomes to all participants.

1.5 DELIVERY, STORAGE, AND HANDLING

A. Delivery and Storage: Deliver materials in original, unopened containers with manufacturer’s labels intact. Protect materials during transport and storage from excessive heat, freezing, and direct sunlight. Store in a cool, dry, and well-ventilated area within a temperature range of 45°F to 95°F.

1.6 WARRANTY

A. Manufacturer’s Limited Warranty: Provide manufacturer’s standard limited warranty for a period of five (5) years from date of installation, covering defects in materials when products are used and applied in accordance with manufacturer’s written instructions.

Warranty is subject to the following conditions:

- Products must be applied within ten (10) months of the date of manufacture.

- Substrates shall be properly constructed and prepared in accordance with industry standards and manufacturer’s recommendations.

- Manufacturer reserves the right to inspect the installation prior to resolution of any warranty claim.

Warranty limitations:
This warranty is limited to replacement of defective material or refund of the original purchase price, at the manufacturer’s discretion, upon verification of a valid claim.

This warranty does not cover:

- Improper substrate preparation

- Installation or workmanship errors

- Cracking, movement, or structural conditions

- Aesthetic variations or natural color changes including variations resulting from hand application techniques and environmental conditions.

- Water intrusion or moisture-related issues

- Labor or costs associated with removal or reinstallation

- Damage caused by misuse, neglect, or acts of nature

This warranty is exclusive and in lieu of all other warranties, whether express or implied. Manufacturer shall not be liable for incidental or consequential damages.

1.7 ENVIRONMENTAL CONDITIONS

• Apply only when ambient and surface temperatures are between 45°F and 95°F
• Do not apply under high humidity or poor drying conditions
• Provide adequate ventilation during application

  - SYSTEM LIMITATIONS

• Lime Wash is a breathable mineral coating intended for decorative purposes only.
• Lime Wash is not a durable or washable finish and is not suitable for high-traffic or high-contact areas.
• Finish may mark, scuff, or exhibit surface wear with contact over time; such conditions are inherent and shall not be considered defects.
• Coating is not intended to resist frequent cleaning or abrasion.
• Substrate conditions, porosity, and application technique will significantly affect final appearance.
• Finish will not conceal substrate imperfections.
• Variations in color, tone, and movement are inherent and expected.

• Lime Wash is not intended for areas subject to frequent contact, cleaning, abrasion, or direct water exposure.

PART 2 - PRODUCTS

2.1 APPROVED MANUFACTURER:

Meoded Paints & Decoration, Inc.

6314 Santa Monica Blvd, Los Angeles, CA 90038

P: 323-308-2600

E: info@meodedpaint.com

www.meodedPaint.com

2.2 SYSTEM PERFORMANCE REQUIREMENTS

A. Material Composition: Lime-based mineral paint composed of slaked lime, mineral pigments, and natural additives formulated to produce a breathable, matte decorative finish. Product shall contain zero (0) VOCs.

B. Vapor Permeability: Coating shall remain breathable, allowing moisture vapor transmission.

C. Appearance Characteristics: Finish shall produce a soft, matte, translucent appearance with natural movement and tonal variation.

2.3 FINISHES

A. Product: Meoded Lime Wash mineral paint system.

B. Finish: Matte, translucent mineral finish with visible movement and tonal variation.

C. Selection:

- Colors shall be selected from the manufacturer's standard palette (MC-701 through MC-899).

- Custom colors available upon request.

- Custom color matching may result in slight variation from submitted samples.

D. Aesthetic Variation: Final appearance shall exhibit variation in tone, movement, shading, and depth inherent to lime wash finishes. Such variation is intentional and shall not be considered a defect.

2.4 ACCESSORIES

A. Primer: Provide Meoded Lime Wash Base Coat or approved mineral primer compatible with lime wash systems.

PART 3 - EXECUTION

3.1 EXAMINATION

A. Verify substrates are suitable for application.
B. Do not proceed until unsatisfactory conditions have been corrected.

3.2 PREPARATION

A. Protect adjacent surfaces not scheduled to receive finish.

B. Substrate Preparation:

- Existing Substrates:

a. Deteriorated surfaces shall be repaired or replaced to provide a sound substrate.

b. Oily, glossy, or previously painted surfaces shall be lightly sanded to promote adhesion.

c. Clean surfaces using appropriate methods to remove dirt, dust, grease, contaminants, and any substances that may interfere with adhesion or appearance.

d. Prime using a manufacturer-approved compatible primer appropriate for the substrate.

Following initial priming, apply two coats of Meoded Lime Wash Base Coat in accordance with manufacturer’s instructions prior to application of finish.

2.   New Substrates:

a. Gypsum wallboard shall have a Level 4 or Level 5 finish and be primed with a PVA primer to achieve a uniform surface.

b. Following initial priming, apply two coats of Meoded LimeWash Base coat in accordance with manufacturer’s instructions prior to application of the finish system.

3.  General Requirements:

a. All substrates shall be clean, dry, and free of dust, dirt, oil, grease, efflorescence, and other contaminants that may affect adhesion or appearance.

b. Substrate shall be suitable to achieve the aesthetic intent of the finish as approved in mock-up and consistent with manufacturer’s recommendations.

c. Priming: Apply Meoded LimeWash Base Coat in accordance with manufacturer’s written instructions.

3.3 MIXING

A. Stir material thoroughly prior to application.
B. Do not dilute material beyond manufacturer’s recommendations.

3.4 APPLICATION

A. Apply in accordance with manufacturer’s instructions.
B. Apply in multiple coats using brush techniques to achieve characteristic lime wash movement.
C. Maintain consistent technique to match approved mock-up.
D. Variations resulting from application technique are inherent to the system.
E. Apply only when ambient and surface temperatures are between 45°F and 95°F, or within manufacturer’s recommended range.

F. Lime Wash is a translucent finish; variations in substrate absorption, application technique, and environmental conditions will significantly influence final appearance.

3.5 CLEANING

A. Remove masking and protection materials.

B. Clean adjacent surfaces without damage.

C. Clean tools immediately after use.

3.6 PROTECTION

A. Protect finished surfaces from damage during construction. Avoid contact until fully cured.

3.7 AESTHETIC INTENT

A. Final finish shall exhibit soft movement, tonal variation, and natural mineral character.

B. Finished work shall be evaluated based on approved mock-up.

C. Lime wash finishes are intentionally non-uniform and designed to develop a natural patina over time.

D. Variations, shading, and visual movement are inherent characteristics and shall not be considered defects.

© Meoded Paints & Decoration, Inc. Document may be copied by design and construction professionals for use in preparing project specifications for Meoded products. Meoded and product names are trademarks of Meoded Paints & Decoration, INC.

Revision Date: April 2026